Information about the DNR’s 30-meter DEM Grid (“demgw930”)

This file last updated on 5/4/02 by John Laedlein, laedlj@dnr.state.wi.us.

The 30-meter (7.5-minute) Digital Elevation Model (“demgw930”) in this directory is an integer grid, with elevation units in meters relative to the National Geodetic Vertical Datum of 1929 (NGVD 29).  ArcView with the Spatial Analyst Extension, ArcInfo/ArcGIS Grid, or comparable GIS software is needed to use the Grid format data meaningfully.

The “demgw930” grid is referenced to Wisconsin Transverse Mercator based on the 1991 adjustment to the North American Datum of 1983 (WTM83/91).  Information about WTM83/91, including projection parameters, is posted on the DNR/EDM website:

http://www.dnr.state.wi.us/org/at/et/geo/wtm8391.html

“Composite” 30-meter DEM Grid (demgw930)

Current DNR 30-meter DEM data holdings include a mix of level2 and "filtered" level 1 DEMs. The DEMs were obtained from USGS over a number of years as 1:24K quad or quarter-quad tiles.  The filtered level1 30-meter DEM tiles, which were obtained from the USGS in January of 2000, were the result of processing that the USGS carried out to improve display characteristics.  Additional information about this filtering is believed to be available on the USGS DEM website.  Throughout the time Lance Perry was here, he would periodically contact USGS or visit their website to see if any improved 30-meter DEM data had become available, acquire the data, then incorporate it into our statewide composite DEM data set in ArcInfo GRID format.  So, what we have currently is a "composite" 30-meter DEM including level2 coverage where it exists, and level1 elsewhere.

Although we share our version of the 30-meter DEMs when requested, we had held off on instituting this as one of our standard datasharing products until we were able to obtain a uniform set of 30-meter DEMs from the USGS (i.e., all Level 2).  However, that did not happen prior to Lance's departure, and we now lack the staff to pursue obtaining and processing newer 30-meter DEMs.  We had deliberately held off on preparing formal metadata for the 30-meter DEMs after statewide holdings at Level2 were available.  In the absence of such metadata, the best available documentation about 30-meter DEMs is the "DEM User Guide" page on the USGS website:

http://edcwww.cr.usgs.gov/glis/hyper/guide/usgs_dem

What are “Level 1” and “Level 2”?

The most common questions about the 30-meter DEMs involve the vertical units and accuracy for the data.  Vertical units are meters.  Vertical accuracy depends on whether the data are level1 or level2, as follows:

"Level-1 DEMs are elevation data sets in a standardized format. The intent is to reserve this level for 7.5-minute DEMs which are created by scanning National High Altitude Photography (NHAP)/NAPP photography. A vertical RMSE of 7 meters is the desired accuracy standard. A RMSE of 15 meters is the maximum permitted."

"Level-2 DEMs are elevation data sets that have been processed or smoothed for consistency and edited to remove identifiable systematic errors. DEM data derived from hyposographic and hydrographic data digitizing, either photogrammetrically or from existing maps, are entered into the Level-2 category. A RMSE of one-half contour interval is the maximum permitted."

# Level Status Information

The directory …\demlib\30m_wtm91\level_status\ includes a shapefile named “quad_status” with some additional information about the demgw930 grid.  The DEM status information in the “quad_status” shapefile was prepared by Kevin Driscoll, WT/2, a mapping specialist working for the Wisconsin Waters Initiative, around July of 2001.

The item “level” indicates whether the corresponding 7.5-minute DEM quad was determined to be Level 1 or Level 2.  According to Kevin, “The means for determining the level of coverage for a particular quad panel is based on the visual clarity of the DEM hillshade for a particular panel compared to the DEM for a particular panel that has been smoothed and edited, identified as level 2.”   The item “CI” indicates the contour interval (in feet) on the corresponding USGS 7.5-minute topographic map.

# The “NED” and the future of DEMs/DTMs

The relationship between the 30-meter DEMs which we have obtained from the USGS and the more recent USGS “National Elevation Dataset” (NED) is unclear.  Depending on staff availability, research may be conducted in the future to determine whether the NED for Wisconsin offers a consistent statewide DEM at level 2 (or an equivalent).  If that were determined, then a WTM83/91 version of the NED could replace our existing “composite” 30-meter DEM.  Eventually, the 30-meter DEMs may be replaced by 10- or 15-meter DEMs when they become available statewide sometime in the future.

In the meantime, more detailed DEMs or Digital Terrain Models (DTMs) continue to be developed on a county-by-county basis as part of the digital orthophoto (DOP) production process.  The DNR EDM and GIS Sections are interested in obtaining these DEMs or DTMs in order to investigate their usefulness and to make the data available to all DNR staff.  If you are involved with a DNR program that has acquired the county-produced DEMs/DTMs, we encourage you to contact us so that the data can be placed in a DNR GIS library for department-wide use.

For additional technical information about the DNR's 30-meter DEM holdings, contact John Laedlein, laedlj@dnr.state.wi.us, or Mitch Moline, molinm@dnr.state.wi.us.